| PROJECT INFORMATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REPORT # |  | DATE (mm/dd/yyyy) /2026 |  |  |  |  | PEMBINA JOB # 2025-6898/4500334658 |  |  |
| CONTRACTOR REED ENERGY GROUP |  |  |  | TORQUE COMPANY REED ENERGY GROUP |  |  |  |  |  |
| PROJECT NAME |  |  |  | LOCATION HYTHE PLANT 11-18-74-12 W6M |  |  |  |  |  |
| CUSTOMER FLANGE/TAG # |  |  |  | DRAWING # |  |  |  |  |  |
| WORK REQUESTED BY PEMBINA PIPELINE CORP. |  |  |  | TORQUE TO PROCEDURE |  |  |  |  |  |
|  |  |  |  | Hot Bolt |  | Other: ESS 00.03.073 |  |  |  |
| TOOL AND MATERIAL INFORMATION |  |  |  |  |  |  |  |  |  |
| FLANGE SIZE 1” |  | FLANGE CLASS 150 |  |  |  |  | FLANGE SERIES (if NPS > 24 in.) |  |  |
| FLANGE FACES |  |  |  | FACE TYPE |  |  |  |  |  |
| Clean | Buffed |  |  | Raised |  |  | Flat |  | RJT |
| METHOD OF FLANGE TIGHTENING |  |  |  | WRENCH (SOCKET) SIZE |  |  |  |  |  |
| Torque | Stud Elongation |  |  |  |  |  |  |  |  |
| LUBRICATION APPLIED JET LUBE 550 |  |  |  | BOLT CONDITION |  |  |  |  |  |
|  |  |  |  | New |  |  | Used |  | Damaged |
| GASKET INSTALLER NAME |  |  |  | GASKET INSTALLER COMPANY REED ENERGY GROUP |  |  |  |  |  |
| GASKET TYPE(S) |  |  |  |  |  |  |  |  |  |
| PRIMARY: CL150 SPIRAL WOUND CGI 316SS |  |  |  | SECONDARY (if Applicable): |  |  |  |  |  |
| TOOL(S) MODEL |  | S/N |  |  |  |  | CALIBRATION DATE (mm/dd/yyyy) |  |  |
| 1. |  | 1. |  |  |  |  | 1. |  |  |
| 2. |  | 2. |  |  |  |  | 2. |  |  |
| 3. |  | 3. |  |  |  |  | 3. |  |  |
| 4. |  | 4. |  |  |  |  | 4. |  |  |
| TORQUE VALUE SPECIFIED (ft/lbs) 40 FT/LBS |  |  |  | MAX HYDROLIC PRESSURE FOR TORQUE SPECIFIED (psi) |  |  |  |  |  |
| STUD MATERIAL SPECIFICATION/GRADE ½” A193 GR. B7M |  |  |  | NUT MATERIAL SPECIFICATION/GRADE A194 GR. 2HM |  |  |  |  |  |
| VERIFICATION CHECKS |  |  |  |  |  |  |  |  |  |
| FLANGE FACES CLEAN AND IN GOOD CONDITION |  |  |  |  | Yes |  | No |  | N/A |
| FLANGE FACES ALIGNED, PARALLEL, NOT FORCED |  |  |  |  | Yes |  | No |  | N/A |
| CORRECT GASKET(S) INSTALLED |  |  |  |  | Yes |  | No |  | N/A |
| CORRECT STUD/NUT SIZE |  |  |  |  | Yes |  | No |  | N/A |
| CORRECT STUD/NUT MATERIAL |  |  |  |  | Yes |  | No |  | N/A |
| CORRECT TIGHTENING SEQUENCE AND TORQUE DETERMINED BASED ON FLANGE CLASS AND SERIES |  |  |  |  | Yes |  | No |  | N/A |
| STUD TIGHTENING SEQUENCE – TORQUE STEPS |  |  |  |  |  |  |  |  |  |
| 1. INITIAL TIGHTENING: Sequence Step 1 (30% of required) |  |  |  |  |  |  |  |  |  |
| VALUE: 12 FT/LBS |  | BY: |  |  |  |  | DATE (mm/dd/yyyy): |  |  |
| 2. SECOND TIGHTENING: Sequence Step 2 (60% of required) |  |  |  |  |  |  |  |  |  |
| VALUE: 24 FT/LBS |  | BY: |  |  |  |  | DATE (mm/dd/yyyy): |  |  |
| 3. THIRD TIGHTENING: Sequence Step 3 (100% of required) |  |  |  |  |  |  |  |  |  |
| VALUE: 40 FT/LBS |  | BY: |  |  |  |  | DATE (mm/dd/yyyy): |  |  |
| 4. VERIFICATION TIGHTENING: Sequence Step 4 (100% of required) |  |  |  |  |  |  |  |  |  |
| VALUE: 40 FT/LBS |  | BY: |  |  |  |  | DATE (mm/dd/yyyy): |  |  |
| 5. FINAL TIGHTENING: Sequence Step 5 (100% of required) |  |  |  |  |  |  |  |  |  |
| VALUE: 40 FT/LBS |  | BY: |  |  |  |  | DATE (mm/dd/yyyy): |  |  |
| DIAGRAM |  |  |  |  |  |  |  |  |  |
| DIAGRAM/REMARKS (attach additional pages if required) |  |  |  |  |  |  |  |  |  |
| REPORT # MARKED ON FLANGE |  |  |  |  | TAG LEFT ON FLANGE |  |  |  |  |
| APPROVAL |  |  |  |  |  |  |  |  |  |
| TORQUE CREW MEMBER (Name) |  |  | SIGNATURE |  |  |  |  | DATE (mm/dd/yyyy) |  |
| PEMBINA PIPELINE CORPORATION INSPECTOR (Name) |  |  | SIGNATURE |  |  |  |  | DATE (mm/dd/yyyy) |  |
| Please email a scanned copy of this completed form to the project manager.; PPL0000 V.01 04-2014. |